Hantek6052BE USB SDK Manual

SDK6052BEDLL.dll (VC++ 6.0)

Define Macro

#define SDK6052BEDLL_HEADFILE_H  //avoid “SDK6052BEDLL.h” file redefinition

#define DLL_API extern "C" __declspec(dllimport)

#define WINAPI __stdcall

Date Type

WORD: unsigned short int

Define Struct

struct PictureRange{	// Be used for draw grid

WORD width; 	// the width of display range

WORD height; 	// the height of display range

WORD left; 	//the left of display range

WORD right;	 //the right of display range

WORD top;	 //the top of display range

WORD bottom; 	//the bottom of display range

};

struct ControlStruct{ 		//save the control data

WORD TriggerSource;	 //the trigger source

WORD SelectedChannel; // the display channel

WORD Timebase; 		// timebase

WORD TriggerPosition;	 // the trigger vertical position “0%,25%,50%,75%,100%”

WORD BufferSize ;		//Its value is 10,512 or 1024

WORD isAlt;			//If ALT trigger, its value is 1, else 0.

};

struct LeversStruct{	//save position

WORD Ch1Position;			// the ch1 waveform horizontal position

WORD Ch2 Position;			// the ch2 waveform horizontal position

WORD Ch1TriggerPosition;		// the ch1 trigger position

WORD Ch2TriggerPosition;		// the ch2 trigger position

WORD EXTTriggerPosition;	// the EXT trigger position

};

struct StateStruct{

WORD Ch1Filt;			// ch1 filter

WORD Ch2Filt;			//ch2 filter

WORD TriggerFilt;		// trigger filter

WORD TriggerMode;	//trigger mode (AUTO, NORMAL, SINGLE)

WORD TriggerSlope;	//trigger slope (rising edge, failing edge)

};

struct LineColor{ 	// the waveform color

WORD R;

WORD G;

WORD B;

};

struct AutosetStruct{		//save the autoset data

WORD WhichChannel;	//which channel will autoset

WORD Ch1Voltage;		//Ch1 autoset voltage

WORD Ch2Voltage;		//Ch2 autoset voltage

WORD Ch1Timebase;	//Ch1 autoset timebase

WORD Ch2Timebase;	//Ch2 autoset timebase

WORD Ch1InGND;		//Ch1 is in GND or not

WORD Ch2InGND;		//Ch2 is in GND or not

WORD Ch1Enabled;		//Ch1 enabled

WORD Ch2Enabled;		//Ch2 enabled

};

struct TimeStruct{		//Save the time

double Timer;		// the number of seconds elapsed since midnight

double Time;		//the time of Timebase

double ScrollTime;

double ScrollStart;

};

Functions:

- Functions for hardware:

  - DLL_API WORD WINAPI sdSearchDevice (WORD DeviceIndex)

Variables introduction:

DeviceIndex:	index of the device.

Note:

This function is used for finding device.If successful return 1, else return 0.

  - DLL_API void WINAPI sdInitLevelRange(WORD DeviceIndex)

Variables introduction:

DeviceIndex:	index of the device.

Note:

This function is used forResetting the relay configuration buffer in the DLL .

  - DLL_API WORD WINAPI sdGetChannelLevel(WORD DeviceIndex,

WORD * pZeroLevel,

WORD nLen)

Variables introduction:

DeviceIndex:	index of the device.

pZeroLevel:	array about the channel adjust Data at Zero Level position.

nLen:		Length of pZeroLevel array.

Note:

This function is used for reading the waveform position adjust data from hardware. If successful return 1, else return 0.

  - DLL_API WORD WINAPI sdSetTriggerAndSampleRate(

WORD DeviceIndex,

WORD Trigger_Slope,

struct ControlStruct *Control_Data1,

WORD TrigFilt)

Variables introduction:

DeviceIndex:	index of the device.

TriggerSlope: 	the trigger type (Rising edge is 0, Failing edge is 1).

Control_Data1: point to variable of structure ControlStruct.

TrigFilt:		trigger filter.

Note:

This function is used for setting trigger and timebase.If successful return 1, else return 0.

  - DLL_API WORD WINAPI sdSetVoltageAndCoupling(

WORD DeviceIndex,

WORD Ch1Voltage

WORD Ch2Voltage,

WORD Ch1_ACDC,

WORD Ch2_ACDC,

WORD TriggerSource)

Variables introduction:

DeviceIndex:	index of the device.

Ch1Voltage: index of the CH1 voltage

Ch2Voltage: Index of the CH2 voltage

Ch1_ACDC: index of the CH1 Coupling ( AC is 1, DC is 0, GND is 0 )

Ch2_ACDC: index of the CH2 Coupling

TriggerSource: index of the trigger source (CH1 trigger is 0, CH2 trigger is 1, ALT trigger is 2, EXT trigger is 3)

Note:

This function is used for setting voltage and Coupling. If successful return 1, else return 0.

  - DLL_API WORD WINAPI sdSetFilt (WORD DeviceIndex,

struct StateStruct *FiltAndTrigger)

Variables introduction:

DeviceIndex:	index of the device.

FiltAndTrigger: point to variable of structure StateStruct

Note:

This function is used for setting channel and trigger filter. If successful return 1, else return 0.

  - DLL_API WORD WINAPI sdSetOffset(WORD DeviceIndex,

struct LeversStruct * Levers,

WORD Ch1_Att,

WORD Ch2_Att,

WORD TriggerSource,

WORD * level)

Variables introduction:

DeviceIndex:	index of the device.

Levers:		point to arriable of the structure LeversStruct.

Ch1_Att :		Ch1 filter.

Ch2_Att :		Ch2 filter.

TriggerSource: index of the trigger source.

level: 		array of the channel position adjust data, read from hardware.

Note:

This if used for adjusting the position of the channel. If successful return true, else return false.

  - DLL_API WORD WINAPI sdCaptureStart(WORD DeviceIndex)

Variables introduction:

DeviceIndex:	index of the device.

Note:

This function is starting to capture data. If successful return 1, else return 0.

  - DLL_API WORD WINAPI sdTriggerEnabled(WORD DeviceIndex)

Variables introduction:

DeviceIndex:	index of the device.

Note:

This function is used for setting the trigger. If successful return 1, else return 0.

  - DLL_API WORD WINAPI sdForceTrigger (WORD DeviceIndex)

Variables introduction:

DeviceIndex:	index of the device.

Note:

This function is used for force trigger. If successful return 1, else return 0.

  - DLL_API WORD WINAPI sdGetData(WORD DeviceIndex,

WORD Ch1_Att,

WORD Ch2_Att,

WORD * Ch1HardwareData,

WORD * Ch2HardwareData,

struct ControlStruct *Control_Data1,

struct StateStruct *FiltAndTrigger,

struct LeversStruct *levers,

WORD *level,

int StartCapture)

Variables introduction:

DeviceIndex:	index of the device.

Ch1_Att :		Ch1 filter.

Ch2_Att :		Ch2 filter.

Ch1HardwareData : save ch1 data from hardware.

Ch2HardwareData : save ch2 data from hardware.

Control_Data1: point to variable of structure ControlStruct.

FiltAndTrigger: point to variable of structure StateStruct

Levers:		point to arriable of the structure LeversStruct.

StartCapture:	capture data flag. If start to capture is 1, else is 0.

Note:

This function is used for reading data from the hardware. If successful return 1, else return 0.

- Functions for software:

  - DLL_API WORD WINAPI sdGetDataSleep(struct ControlStruct *Control_Data1)

Variables introduction:

Control_Data1: point to variable of structure ControlStruct.

Note:

This function is used for Get necessary Delay time. If successful return 1, else return 0.

  - DLL_API WORD WINAPI sdMainPanelDrawGrid (HDC hdc,

struct PictureRange * view_data)

Variables introduction:

Hdc:			resource Handle //  handle to the display picture.

view_data:	Information about drawing grid.

Note:

This function is used for drawing grid on panel. If successful return 1, else return 0.

2.3	DLL_API WORD WINAPI sdGetScrollData(WORD DeviceIndex,

struct ControlStruct *Control_Data1,

struct LeversStruct *levers,

struct TimeStruct *TimeState,

WORD Ch1Voltage,

WORD Ch2Voltage,

WORD *Ch1ScrollData,

WORD *Ch2ScrollData,

WORD *Ch1DisplayData,

WORD *Ch2DisplayData,

WORD TriggerPosition,

bool Ch1InGND,

bool Ch2InGND)

Note:

This function is used for getting and processing data when channel is in scroll mode. If successful return 1, else return 0.

2.4	DLL_API double WINAPI sdDisplaySampling (bool Ch1Enabled,

bool Ch2Enabled,

WORD TriggerSource,

WORD Timebase,

WORD BufferSize)

Note:

This function is use for Display Sampling on main panel status bar.

2.5	DLL_API WORD WINAPI sdChannelDataBuffer (struct ControlStruct *Control_Data1,

WORD InterpolationLineMode,

WORD * chx_data,

WORD * ch_data)

Variables introduction:

InterpolationLineMode:	//its value is 1, 0, or 2

Note:

This function is used for processing data and copying data to “chx_data” from “ch_data”. If successful return 1, else return 0.

2.6	DLL_API WORD WINAPI sdGetDisplayData(struct ControlStruct *Control_Data1,

WORD * ChDispalyData,

WORD * ChBufferData)

Variables introduction:

ChDisplayData:		//save Channel display data

ChBufferData:		//save Channel data

Note:

This function is used for getting display data. If successful return 1, else return 0.

2.7	DLL_API WORD WINAPI sdDisplayData (HDC hdc,

struct PictureRange * DisPicture,

WORD PointOrLine,

WORD * ChDisplayData,

struct LineColor * ch_color)

Note:

This function is used for displaying data on screen.

2.8	DLL_API WORD WINAPI sdChannelInGND (WORD Ch1Position,

WORD *chdata)

Note:

This function is used for processing data when channel is in GND. If successful return 1, else return 0.

2.9	DLL_API WORD WINAPI sdAutoset (WORD DeviceIndex,

struct ControlStruct *Control_Data1,

struct AutosetStruct *AutosetData,

struct StateStruct *FiltAndTrigger,

struct LeversStruct * levers,

WORD *level

)

Note:

This function is used for automatically setting. If successful return 1, else return 0.

  - DLL_API WORD WINAPI sdSoftFindTriggerPoint(WORD nHTriggerPosition,

WORD TIMEBASE,

WORD TriggerLever,

WORD TriggerSlope,

WORD * ChBufferData)

Note:

This function is used for finding trigger position by software and getting needed channel data. If successful return trigger position finded by software.

  - LL_API WORD WINAPI sdSoftFindTriggerPointCopy(

WORD nHTriggerPosition,

WORD TriggerPoint,

WORD *ChBufferData)

Note:

This function is used for getting needed channel data according to the value TriggerPosition. If successful return 1.

- Flow Chat: